# Effects of fuel composition on the minimum ignition energy and its transition for homogeneous biogas-air mixtures

V. Papapostolou1, C. Turquand d’Auzay1, S.F. Ahmed2, N. Chakraborty1

1 School of Engineering, Newcastle University, Newcastle-Upon-Tyne NE1 7RU, United Kingdom

2Department of Mechanical and Industrial Engineering, College of Engineering,

Qatar University, P. O. Box 2713, Doha, Qatar

Abstract

The minimum ignition energy (MIE) requirements for ensuring successful ignition, and self-sustained flame propagation following external energy deposition have been analysed for the forced ignition of a homogeneous stoichiometric biogas-air mixture for a wide range of initial turbulence intensities and CO2dilutions using three-dimensional Direct Numerical Simulations (DNS) under decaying turbulence. The biogas is represented by a methane-carbon dioxide fuel blend, and the amount of carbon dioxide present in the fuel blend has been varied so it is comparable to those encountered in industrial settings. The present study illustrates that an increase in CO2 dilution has a significant adverse effect on the ignition, due to the heatsink effect of CO2. The variation of the MIE normalized by its respective laminar value with root-mean-square turbulent velocity fluctuation exhibits self-similar behaviour irrespective of the level of CO2 dilution.

# Introduction

Localised forced ignition (e.g. spark or laser ignition) of homogeneous mixtures is of critical importance for the efficient use of fuel in Spark Ignition (SI) engines and industrial gas turbines. The eco-friendly alternative renewable fuels are becoming ever more important because of limited reserves of conventional fossil fuels. One such alternative fuel is biogas, which can be used as either a complement or a replacement for existing fossil fuels. Biogas is primarily composed of methane (CH4) and carbon dioxide (CO2). The composition of biogas is off critical importance, as large variations in CO2 content can affect ignition, leading to adverse effects on the subsequent flame propagation.

Due to the fundamental importance of localised forced ignition, a number of experimental [1-4] and computational [5,6] studies have investigated this phenomenon. In-depth experimental studies by Shy et al. [1] analysed the minimum ignition energy (MIE) for homogeneous methane-air mixtures under homogeneous isotropic forced turbulence, across varying equivalence ratios and turbulence intensities, u'/sl0 (where u' is the root-mean-square (rms) value of turbulent velocity fluctuations, and sl0 is the unstrained laminar burning velocity). A transition in the increase of the MIE with increasing turbulence intensity was observed at a critical value uc'/sl0. For u'/sl0<uc'/sl0 the MIE requirement increases gradually and is significantly smaller than the MIE for u'/sl0>uc'/sl0, where the MIE requirement increases rapidly with increasing u', for u'/sl0>uc'/sl0 and this transition has been justified based on scaling arguments presented by Shy et al. [1] and Turquand d’Auzay et al. [6]. Subsequently, more recent experimental studies by Cardin et al. [2] have investigated MIE variation using laser ignition for lean methane-air mixtures under isotropic homogeneous decaying turbulence, and observed a similar behaviour and transition of the MIE, that Shy et al. [1] observed using a spark ignition system. Thus, it can be inferred that the behaviour and transition of the MIE is independent of the ignition system and fuel used. Larsson et al. [3] experimentally investigated the MIE required for successful ignition for methane and propane mixtures separately diluted with nitrogen and carbon dioxide, with a view towards finding the maximum amount of inert gas concentration in a given gas composition which could be successfully ignited. The turbulence and spark characteristics where kept unaltered across all the experiments carried out and it was observed that as the volumetric percentage of diluent (CO2) in the fuel blend increased, so did the MIE for the biogas mixture. It was found that successful ignition was possible up to a maximum of 25% carbon dioxide volumetric dilution in the fuel blend (CH4+CO2). Galmiche et al. [4] investigated the effects of carbon dioxide dilution (5-40% by volume) on localised forced ignition, and reported that the carbon dioxide acted as a heat sink, and an increase in the amount of CO2 present in the mixture, leads to an increase of the energy required for a successful ignition. It was also observed that biogas-air mixtures show a slower combustion process when compared to methane-air mixtures, which is attributed to the lower burning velocity of biogas-air mixtures.

Three-dimensional Direct Numerical Simulations where utilized to analyse the early stages of premixed flame kernel development, stemming from localized forced ignition [5,6]. The negative effects of turbulence intensity in regards to successful ignition have been numerically replicated by Turquand d’Auzay et al. [6], and it was reported that an increase of turbulence intensity with a fixed length scale has a detrimental effect on successful ignition. An in-depth review on localised forced ignition for both homogeneous and inhomogeneous mixtures was published by Mastorakos [7,8], and interested readers are directed there for further information. Of particular interest to this study is the transition of MIE from low to high turbulence intensities observed across a wide range of fuels, that was reported by Shy et al. [1] and Cardin et al. [2], and was subsequently numerically replicated by Turquand d’Auzay et al. [6] for premixed stoichiometric methane-air mixtures under decaying homogeneous isotropic turbulence. To the best of the authors knowledge, the MIE variation dependent on volumetric percentage of diluent present in compound fuels such as biogas has not been investigated. The present study aims to address this gap in the existing literature by utilising three-dimensional compressible DNS to investigate the variation of the MIE for premixed stoichiometric homogeneous CH4-CO2-air mixtures under decaying isotropic turbulence, with varying levels of CO2 dilution. The MIE has been evaluated for, solely a successful ignition and also a successful ignition followed by self-sustained combustion once the ignitor has been switched off. Thus, the objectives of the present study are to understand the effects that varying levels of dilution of the fuel with CO2 have on the variation of the MIE requirements for successful ignition and for subsequent flame propagation under different initial intensities of homogeneous isotropic decaying turbulence for different levels of CO2 dilution.

# Mathematical Backgound

The combustion of biogas can be represented by the following global equation:

CH4+2O2+3.76N2+a⋅Diluent→

CO2+2H2O+7.52N2+ a⋅Diluent        (1)

where the diluent is taken to be CO2 for the present work. The biogas composition is described following the definition presented by Galmiche et al. [4] as ψu=a/(a+10.52) where ψu represents the dilution percentage which is the molar fraction of the diluent (CO2) in the unburned gases. To account for the presence of CO2 and in the interest of computational economy, a two-step chemical mechanism presented by Westbrook and Dryer [9] has been used for the present analysis. The chemical mechanism used, accounts for 6 species (CH4, O2, CO2, H2O, CO and N2) and is a compromise between single-step chemistry and more complex mechanisms. The above chemical mechanism has been validated for both CH4 and CH4+CO2 mixtures, against other chemical mechanisms (GRI-Mech 3.0 [10] and 2s_CM2 [11]) and experimental results [4], and for further details regarding the performance and validation of the mechanism, interested readers are directed to recent work by Turquand d’Auzay et al. [5]. It must be mentioned that the present chemical mechanism overestimates the flame speed for all values of ψu, which is not expected to affect the ignition phenomenon, but is expected to have minor implications on subsequent self-sustained propagation, and thus the present study has limited itself to an intermediate level of diffusion, with ψu≤0.10, for which flame speed predictions are sufficiently accurate [5].

In line with previous studies [5-8] on localised forced ignition, several assumptions are made for the present study. All species in the gaseous phase are taken to be ideal gases, and thus the ideal gas law is used for compressible flow DNS. Standard values were taken for the ratio of specific heats (γ=CPg/Cvg=1.4, where Cpg and Cvg are the gaseous specific heats at constant pressure and volume, respectively) and Prandtl number (Pr=μCpg/λ=0.7, where μ is the dynamic viscosity and λ is the thermal conductivity of the gaseous phase). The species diffusion velocity is accounted for by using Fick’s law of diffusion, where the transport properties are similar and independent of temperature across all species. To account for the effects of the localised forced ignition, an additional source term, (q''' = Asp exp(-r2/2Rsp2) with r being the distance from the ignitor centre and Rsp representing the characteristic width of energy deposition) is added to the energy conservation equation, which follows a Gaussian distribution in the radial direction from the ignition centre [5-8]. The constant Asp is determined by a volume integration which leads to the total ignition power  Q given by [5-8]:Q=Vq''' dV= asp ρ0Cpτ T043π δz03Ht-Ht-tsp /tspwhere asp is a parameter determining the total energy deposited by the ignitor, τ = (Tad0-T0)/T0 is the heat release parameter (where Tad0 and T0 are the adiabatic flame temperature of the undiluted stoichiometric mixture and unburned gas temperature respectively), δz0 is the Zel'dovich flame thickness of the undiluted stoichiometric mixture (δz0 = D0/sl0, where D0 is the unburned mass diffusivity of the reactants) and Ht, and Ht-tsp are Heaviside functions, which ensure that the ignitor is only active until t = tsp. The energy deposition duration tsp, is expressed as tsp = bsptf, where tf = δz0/sl0 is a characteristic timescale and the energy deposition parameter bsp is given a value of bsp = 0.2 in the present study, which falls within its optimal range (0.2≤bsp≤0.4) [5-8]. The details of the spark formation (momentum modification contribution, plasma and shock wave formation) are not considered in this analysis for the purpose of simplicity and computational economy. In the present study, bsp and Rsp are kept constant (i.e. bsp=0.2, Rsp=2.45δz0) whilst asp is varied until the minimum energy levels leading to either of the two following phenomena are found:

1. Ignition - A successful ignition refers to a situation where the maximum temperature surpasses the adiabatic flame temperature (Tadψ) during or after the energy deposition period regardless of subsequent flame behaviour. If the maximum temperature does not reach the adiabatic flame temperature, it is referred to as a misfire in the following discussion.

2. Self-sustained propagation - A successful self-sustained propagation is obtained when the flame kernel burns without the aid of the ignitor after a successful ignition. It is determined by evaluating the temporal evolution of the burned gas volume, that is, if the temporal derivative is positive at the end of the simulation time.

# Numerical Implementation

The simulations have been carried out using the three-dimensional compressible DNS code SENGA+ [5,6] in two separately sized domains depending on whether the MIE for either ignition or propagation was being determined. For ignition across all turbulence intensities investigated, the computational domain is taken to be 44δz0×44δz0×44δz0 or 4.8lt×4.8lt×4.8lt, and subsequently when self-sustained propagation was investigated a larger domain of 66δz0×66δz0×66δz0 or 7.4lt×7.4lt×7.4lt was used. The domain sizes have been chosen in such a manner to accommodate the development of the kernel in the computational domain for as long as possible. The domains are discretized by Cartesian grids of 352×352×352and  540×540×540, respectively, with cells of uniform size Δx, which ensures 10 grid points across the undiluted thermal flame thickness δth0  =Tad0- T0max∇ TL<δthψu. It also ensures ηk >Δx, where ηk is the Kolmogorov length scale. The spatial differentiation and time advancements have been carried out using high order finite difference and explicit Runge-Kutta schemes respectively [5,6]. The flame-turbulence interaction takes place under decaying isotropic homogeneous turbulence. A well-known pseudo-spectral method is used to initialise the turbulent velocity fluctuation by an incompressible, homogeneous isotropic field with prescribed values of root-mean-square (rms) values (u') and integral length scale (lt). The initial integral length scale is kept constant throughout the study at lt/δz0=9, and it remains comparable with previous computational studies of localised ignition [5,6].

Figure 1. The cases investigated here on the regime diagram. The cases with ψu=0.025,0.05 and 0.10 are shown by black, red, blue and magenta points.

The present study investigates the ignition phenomenon across three separate levels of dilution ψu=0.025,0.05 and 0.10, which for brevity are referred to as DIL02, DIL05 and DIL10, respectively. The mixture across all cases investigated remains perfectly premixed and stoichiometric and the initial mass fractions of oxidizer, methane and carbon dioxide can be recovered for each dilution percentage investigated from the definition of ψu in conjunction with Eq. 1. Figure 1 shows all the cases investigated in the present study on the regime diagram. Additionally, the cases investigated by a previous computational study [5] on the forced ignition of undiluted stoichiometric methane-air mixtures, are included to allow for comparisons between the studies. The ratio lt/δz0 is kept constant, however when the respective diluted mixture value δzψ is used, the ratio lt/δzψ decreases with increasing dilution percentage as shown in Fig. 1, as the presence of CO2 thickens the flame.

Figure 2. - Variation of u'/slψ with u'/sl0 (dots), and variation of lt/δzψ with lt/δz0 (lines), across all cases investigated.

Figure 3. Temporal evolutions of the maximum value of Tnorm under laminar conditions with ψu=0.025, 0.05 and 0.10 as indicated by the legend for an input energy equal to the MIE for ψu=0.025 under laminar conditions.

The cases investigated here were set up to mimic combustion occurring in a combustion chamber of a gas turbine, where the initial turbulence intensity (u') is determined beforehand and subsequently remains unaltered, whilst the level of dilution in the fuel is subject to change, due to the difficulty of producing industrial quantities of biogas with fixed composition (ψu), and this approach is consistent with the experimental work by Larsson et al [3]. This in turn means that even as u' and lt are kept constant, as the mixture composition varies, the ratios of u'/slψ and lt/δthψ vary as shown in Fig. 2, and this has a significant effect on ignition and subsequent flame behaviour. The DNS simulations, which are used to evaluate the MIE only for successful ignition, have been carried out for at least t=2tsp, whilst the simulations for successful propagation, have been carried out till t=10tsp, or until the kernel exits the domain. For each turbulence intensity across all dilution percentages investigated, multiple simulations have been carried out with different values of the ignition energy parameter asp to estimate the MIE to a precision of about 1% of the corresponding diluted laminar MIE value for ignition, whilst for propagation the MIE was sought with an accuracy of 2-4% of the corresponding diluted laminar MIE value.

Tnorm=0.1 Tnorm=0.3 Tnorm=0.5 Tnorm=1.0 Energy deposition region Tnorm=0.1 Tnorm=0.3 Tnorm=0.5 Tnorm=1.0 Energy deposition region

Figure 4. Isosurfaces of (grey) Tnorm=0.1, (green) 0.3, (orange), 0.5 and (red) 1.0 with the energy deposition region indicated by the translucent red sphere, obtained for ΓMIEψui with u'/slψ≈10.50 for ψu=0.025 (left column) and ψu=0.10 (right column). Kernels are shown at t/tsp=0.5, 1.0, 1.5, 2.0, 2.5 (top to bottom).

# Results and Discussion

To understand the effect that varying levels off dilution have on the ignition phenomenon, it is instructive to examine the temperature profiles under laminar conditions across the different dilution levels investigated. To illustrate the effects of increasing ψu, the temporal evolutions of the maximum value of Tnorm=(T-T0)/(Tadψ-T0) under laminar conditions for three dilution percentages are shown in Fig. 2 for an input ignition energy which is the MIE for ψu=0.025 An increase of ψu leads to a decrease in the maximum value of Tnorm. It can be seen from Fig. 2 that thermal runaway takes place only for ψu=0.025, and the maximum value of Tnorm surpasses its value   corresponding to the respective adiabatic flame temperature (Tnorm>1.0) at t/tsp=4.0, whilst for the other dilution percentages Tnorm decreases after the end of the energy deposition duration (t/tsp=1.0), and results in a failed ignition. The temporal evolution of (Tnorm)max for the case with ψu=0.025 in Fig. 3 is indicative of an auto-ignition, as the thermal runaway happens after the end of energy deposition duration, and this behavior is consistent with previous findings [6]. Due to all three different cases being laminar and identical energy being deposited into the mixture, as expected a higher ψu can lead to unsuccessful ignition, highlighting the ‘heat sink effect’ of CO2 which is also consistent with a recent computational analysis by Turquand d’Auzay et al. [5]. This in turn suggests that the MIE requirement increases as ψu increases whilst u'/sl0is constant, and this has been observed by the present study and is discussed later in this paper. Thermal runaway due to autoignition after the energy deposition period has also been observed for the MIE corresponding to ψu=0.05 and 0.10 under laminar conditions, and the temporal evolutions of (Tnorm)max for the MIE for ψu=0.05 and 0.10 are qualitatively similart to the profile shown for ψu=0.025 in Fig. 3. It is also worth mentioning how the time delay (≈4tsp) for (Tnorm)max  to surpass the adiabatic flame temperature (i.e. Tnorm=1.0) for the case shown in Fig. 3, is nearly double in comparison to the time delay observed for undiluted methane air mixtures (≈2.3tsp)  under identical conditions (see Fig. 5a from [6]). This illustrates the detrimental effects of CO2dilution on successful ignition, along with the slower combustion process that arises when compared to undiluted methane-air mixtures, which is in agreement with experimental studies [4].

To further investigate the effects of varying ψu on ignition in conjunction with initial turbulence intensity, isosurfaces of different values of Tnorm at t/tsp=0.5,1.0,1.5,2.0,2.5 are presented in Fig. 4. Two cases with almost identical u'/slψ (≈10.50) with ψu=0.025 and 0.10 were chosen for the purpose of comparison. From Fig. 2 it can be seen that despite u'/slψ being nearly the same, for the case with ψu=0.025 it corresponds to u'/sl0=9.5, whilst for ψu=0.10 it amounts to u'/sl0=6.0. Previous studies have shown that a higher u'/sl0 has an adverse effect on ignition [1,2,6-8,12,13]. Observing the temporal evolution of the Tnorm isosurfaces shown in Fig. 4, it can be seen that despite u'/slψ being very similar, the ignition kernel with ψu=0.025 is significantly more wrinkled and elongated due to turbulence than the corresponding ψu=0.10 case. At t=2.0tsp, both cases have achieved successful ignition as an isosurface of Tnorm=1.0 can be seen. However, at t=2.5tsp for the case with ψu=0.025, the kernel is quenching as the Tnorm=1.0 isosurface is not present anymore, whilst for the case with ψu=0.10 the Tnorm=1.0 isosurface is growing, indicating combustion is continuing at that stage. This result is superficially counter-intuitive but it is important to also account for the level of energy deposited. In order to allow for direct comparison between cases with different dilution percentages, the MIE is normalised based on the MIE obtained for the respective ψu under laminar conditions, and is calculated asΓMIEψui=MIEψu/MIE0ψu, where the superscript ψu indicates a constant dilution level, whilst the subscript 0 indicates laminar conditions. For the case with ψu=0.025, ΓMIEψu is 1.085 (i.e. ΓMIEψui=1.085), whilst ΓMIEψui=1.025 is obtained for the case with ψu=0.10. the MIE0ψufor the ψu=0.10 case is comparable to that in the ψu=0.025case (e.g.MIE0ψu[ρ04πδz033)CpTad0-T0=3.85 and 3.90 for ψu=0.025 and 0.10 respectively).

Propagation ΓMIEψu∝u'sl02 Ignition ΓMIE∝u' sl00.24 Ignition & Propagation ΓMIEψu∝u'sl00.03 ΓMIE∝u'sl00.2 Propagation ΓMIEψu∝u'sl02 Ignition ΓMIE∝u' sl00.24 Ignition & Propagation ΓMIEψu∝u'sl00.03 ΓMIE∝u'sl00.2

Figure 5. Variations of normalised MIEs (a) aspMIEα/p (dots) and propagation (diamonds and dashed line) and (b) ΓMIEψui/pwith u'/sl0.

Thus, the resulting kernel ignites and then subsequently quenches for ψu=0.025, whilst the case with ψu=0.10 exhibits  successful ignition and the kernel continues to grow up to the time instant shown in Fig. 4 for almost similar ignition energy input. From the above observations, it can be surmised that although the level of CO2 dilution in the mixture plays a significant role in whether successful ignition will occur, u'/sl0 remains the dominating factor (e.g. u'sl0=9.5 and 6.0 for ψu=0.025 and 0.10 respectively) because an increase in u', for a given value of lt increases the eddy thermal diffusivity Dt~u'lt  and thus increases the heat transfer rate from the hot gas kernel and acts to quench (or decrease) the kernel (burning rate), as shown by previous studies [1,2,6-8,12,13]. To further investigate this effect it is instructive to examine the variations of aspMIEα=MIEψuα[ρ04πδz033)CpTad0-T0 and ΓMIEψu α (with α=i  for successful ignition and p for successful self-sustained flame propagation) as a function of u'/sl0 in Fig. 5. The corresponding variations of aspMIEα and ΓMIEψu α as a function of u'/slψ are shown in Fig. 6.

Propagation ΓMIEψu∝u'slψ2 Ignition ΓMIE∝u'slψ0.24 Ignition & Propagation ΓMIE∝u'slψ0.2 Propagation ΓMIEψu∝u'slψ2 Ignition ΓMIE∝u'slψ0.24 Ignition & Propagation ΓMIE∝u'slψ0.2

Figure 6. Variations of normalised MIEs (a) aspMIEα/p (dots) and propagation (diamonds and dashed line) and (b) ΓMIEψui/pwith u'/slψ.

It can be seen from Figs. 5a that (aspMIE)i increases with increasing ψu and this behaviour is consistent for all u'/sl0 . This is consistent with previous analysis [6] and observations made in the context of Fig. 3. The increase in the energy requirement with increasing ψu is attributed to the ‘heat sink effect’ of CO2, as a higher dilution level corresponds to more CO2 in the mixture. A higher energy level is accordingly required for larger value of ψu so that the heat release rate in the hot gas kernel can supersede the heat transfer rate to give rise to eventual thermal runaway in the cases of successful ignition.  For a given set of initial values of u'/sl0 and lt/δz0, the eddy diffusivity remains mostly  unaffected by the values of ψu, and thus the driving factor for the higher MIE requirement  is the reduction in chemical heat release rate with increasing ψu. Figure 5b, where ΓMIEψu is plotted as a function of u'/sl0, which shows that ΓMIEψu increases with increasing u'/sl0 and  exhibits a transition of the MIE requirement at around uc'/sl0=8.0 as indicated by the trend lines. The normalised MIE requirements for self-sustained flame propagation (aspMIE)p and ΓMIEψup  for ψu=0.10 is also exemplarily shown in Figs. 5b and 6b. It can be seen from Figs 5b that ΓMIEψupshows a transition at uc'sl0=6 and the MIE requirement increases rapidly for u'/sl0>uc'/sl0. However, these transitions of (aspMIE)p and ΓMIEψui are obtained for different values of u'/slψ for different values ψu. Moreover, the increased demand of MIE for higher value of ψ is not clearly evident from Fig. 6a.

These differences arise due to the differences between the values of u'/slψ  and u'/sl0, as shown in Fig. 2 and thus two identical values of u'/slψ for different ψu values correspond to different values of u'/sl0. The variations of ΓMIEψup with u'/sl0 and u'/slψ exhibit a power-law with an exponent of 2.0 for u'/sl0>uc'sl0, as shown by the trendlines in Figs. 5b and 6b. This is quantitatively consistent with previous computational findings based on undiluted stoichiometric CH4-air mixtures [6] and the experimental results by Cardin et al. [2]. A qualitatively similar MIE (for self-sustained flame propagation) variation with turbulence intensity with much larger power-law exponent has been reported by Shy et al. [1]. The possible differences between the numerical results and findings by Shy et al. [1] have been discussed in Ref. [6] and thus are not repeated here. Turquand d-Auzay et al. [6] also explained ΓMIEψup~u'sl02 based on a scaling argument, which was in good agreement for the undiluted stoichiometric CH4-air mixture. The present results indicate that the scaling ΓMIEψup~u'sl02 may also be valid for biogas-air mixtures. The increasing demand of the MIE with an increase in u' is in good qualitative agreement with both previous experimental [1,2,7,8] and computational [6,12,13] analyses. Moreover, a comparison between (aspMIE)p  and (aspMIE)i for ψu=0.1 reveals that the MIE for self-sustained flame propagation is likely to be greater than the MIE for just successful ignition for u'/sl0>uc'/sl0 , which is also consistent with previous findings [6] and interested readers are referred to Turquand d’Auzay et al. [6] for the relevant physical explanation. Of particular interest is the collapse of all the ΓMIEψu curves for the ψu values investigated here into one. As outlined previously an increase of ψu results in a higher energy requirement, but ψu does not affect the ΓMIEψui variation with u'sl0, and ΓMIEψui is solely dependent on u'/sl0. This in turn implies that once the MIE for a given ψu under laminar conditions is determined, it is possible to estimate the ΓMIEψu required for any u'/sl0 following the trend line shown in Fig. 5b.

# Conclusions

The minimum ignition energy (MIE) of stoichiometric biogas-air mixtures with varying levels of CO2 dilatation have been numerically evaluated under homogeneous isotropic decaying turbulence for a wide range of initial turbulence intensities. An increase in mole fraction of CO2 in the CH4/CO2 blend (ψu) increases the MIE demand significantly which is in good qualitative agreement with previous experimental results [4]. It was also shown that the variation of the MIE for successful ignition normalised by its respective laminar value ΓMIEψuiwith u'/sl0collapses into a single distribution for all levels of dilution, which allows for the MIE at a given u'/sl0 to be calculated with an acceptable level of accuracy for a range of different homogeneous biogas-air mixtures. The variation of the MIE with turbulence intensity exhibits a transition such that the energy demand increases significantly above a threshold value of turbulence intensity for both successful ignition and self-sustained flame propagation. This behaviour has been found to be consistent with previous numerical [6] and experimental [1,2] findings. However the critical u'slψat which the transition occurs appears to be dependent on ψu but the corresponding u'sl0 value remains mostly unaffected by CO2 dilution. The MIE for self-sustained propagation assumes considerably higher values than the MIE for only successful ignition and a scaling for the normalised MIE for self-sustained flame propagation ΓMIEψup for undiluted stoichiometric methane-air mixture has been found to be valid even for the stoichiometric biogas-air mixture with ψu=0.1. Further investigation will be necessary to analyse the possibility of the self-similar behaviours of ΓMIEψup  with the variation of  u'sl0.

# Acknowledgements

The authors would like to thank the British Council, ESPRC (EP/K025163/1, EP/R029369/1) Rocket (HPC facility at Newcastle University), Cirrus and ARCHER for financial and computational support

# References

[1] S.S. Shy, C.C. Liu, W. Shih, Combust. Flame 157 (2) (2010) 341–350.

[2] C. Cardin, B. Renou, G. Cabot, A.M. Boukhalfa, Comptes Rendus Mécanique 341 (1–2) (2013) 191–200.

[3] A. Larsson, A. Berg, A. Bonaldo, Proc. ASME Turbo Expo, (2013)  GT2013-95536.

[4] B. Galmiche, F. Halter, F.  Foucher, F., P.  Dagaut, Energy Fuels, 25(3) (2011) 948–954

[5] C. Turquand d’Auzay, V. Papapostolou, S.F. Ahmed, N. Chakraborty, Combust. Sci. Technol. (2019) (accepted)

[6] C. Turquand d’Auzay, V. Papapostolou, S.F. Ahmed, N. Chakraborty, Combust. Flame, 201 (2019) 104–117.

[7] E. Mastorakos, Prog. Energy Com- bust. Sci. 35 (1) (2009) 57–97.

[8] E. Mastorakos, Proc. Combust. Inst. 36 (2) (2017) 2367–2383.

[9] C.K. Westbrook, F.L. Dryer, Combust. Sci. Technol., 27(1–2) (1981) 31–43.

[10]http://www.me.berkeley.edu/grimech/version30/text30.html (2000).

[11] J. Bibrzycki, T. Poinsot, Work. note ECCOMET WN/CFD/10/17, CERFACS (2010).

[12] D. Patel, N. Chakraborty, Int. J. Spray Combust. Dyn., 7 (2015) 151-174.

[13] D. Patel, N. Chakraborty, Int. J. Spray Combust. Dyn. 8, (2016) 183-196.